pfSense Network Segmentation

Homelab VLAN Architecture & Firewall Design

Platform: pfSense CE 2.8.1  |  Hypervisor: VMware Workstation  |  Date: April 2026

# 1. Project Overview

This project demonstrates enterprise-grade network segmentation using pfSense Community Edition deployed as a virtual machine on VMware Workstation. The objective was to design, implement, and document a multi-VLAN architecture that enforces security boundaries between device categories — a fundamental skill in network engineering and security operations.

The architecture separates network traffic into four isolated segments, each with explicitly defined firewall rules and documented business justification. This mirrors real-world enterprise deployments where different device types carry different risk profiles and must be segregated accordingly.

# 2. Network Architecture

## 2.1 Infrastructure Overview

pfSense acts as the core firewall and router, sitting between the WAN (internet-facing) interface and a managed virtual switch. All VLANs are trunked over a single LAN interface (em1) using 802.1Q tagging. Each VLAN is presented to pfSense as a separate logical interface with its own subnet, DHCP scope, and firewall ruleset.

| VLAN ID | Name | Subnet | DHCP Range | Purpose |
| --- | --- | --- | --- | --- |
| VLAN 10 | Trusted | 192.168.10.0/24 | .100 – .199 | PCs, laptops, admin devices |
| VLAN 20 | IoT | 192.168.20.0/24 | .100 – .199 | Smart devices, cameras |
| VLAN 30 | Guest | 192.168.30.0/24 | .100 – .199 | Visitor WiFi, internet only |
| VLAN 40 | Servers | 192.168.40.0/24 | .100 – .199 | Homelab VMs, NAS |

## 2.2 Interface Assignments

Each VLAN is assigned as a separate pfSense interface (OPT1–OPT4) with a static IPv4 address serving as the default gateway for that segment. DHCP servers are enabled on each interface, ensuring devices receive addresses in the correct range automatically.

# 3. Firewall Rule Design

## 3.1 Design Principles

All firewall rules follow a default-deny model — traffic is blocked unless explicitly permitted. Rules are evaluated top-to-bottom on each interface; the first matching rule wins. This architecture enforces the principle of least privilege: each VLAN has access to only what it needs to function.

## 3.2 Rule Summary by Interface

TRUSTED (VLAN 10)

| # | Source | Destination | Action | Business Justification |
| --- | --- | --- | --- | --- |
| 1 | TRUSTED subnets | 192.168.0.0/16 | BLOCK | Prevents trusted devices from reaching IoT or Guest VLANs — limits blast radius if a trusted device is compromised |
| 2 | TRUSTED subnets | Any | PASS | Allows trusted users full internet access for productivity |

IoT (VLAN 20)

| # | Source | Destination | Action | Business Justification |
| --- | --- | --- | --- | --- |
| 1 | IoT subnets | 192.168.0.0/16 | BLOCK | IoT devices have a poor security track record. A compromised smart device must not be able to reach internal servers or trusted endpoints. |
| 2 | IoT subnets | Any | PASS | IoT devices require internet access for firmware updates and cloud services. |

Guest (VLAN 30)

| # | Source | Destination | Action | Business Justification |
| --- | --- | --- | --- | --- |
| 1 | GUEST subnets | 192.168.0.0/16 | BLOCK | Guest users are untrusted by definition. No visibility into internal network is permitted under any circumstance. |
| 2 | GUEST subnets | Any | PASS | Guests require internet access only. Strict isolation is maintained. |

Servers (VLAN 40)

| # | Source | Destination | Action | Business Justification |
| --- | --- | --- | --- | --- |
| 1 | SERVERS subnets | 192.168.0.0/16 | BLOCK | Servers should not initiate connections to client VLANs. Outbound server traffic to internal networks indicates possible compromise or misconfiguration. |
| 2 | SERVERS subnets | Any | PASS | Servers require internet access for updates, package downloads, and external API calls. |

# 4. Security Rationale

Each design decision in this architecture reflects a real-world security principle:

- Default deny posture — no traffic is permitted unless explicitly allowed. This forces every rule to have a deliberate purpose.

- IoT isolation — IoT devices historically have poor patch cycles and default credentials. Isolating them means a compromised smart device cannot pivot to servers or trusted endpoints. The blast radius is contained to the IoT VLAN.

- Guest isolation — guest users are treated as untrusted by definition. Complete isolation prevents any reconnaissance of internal systems.

- Server outbound restriction — blocking server-to-VLAN traffic prevents lateral movement if a server is compromised. A compromised web server should not be able to reach internal workstations.

- Documented justifications — every rule has a written business reason. This mirrors enterprise change management practices and demonstrates security engineering thinking, not just configuration.

# 5. Technical Configuration Summary

## 5.1 Platform

pfSense CE 2.8.1 running on FreeBSD 15.0, deployed as a VMware Workstation virtual machine with 8 GB RAM and 2 vCPUs. WAN interface uses VMware NAT (em0). LAN interface uses VMware Host-only networking (em1) with all VLANs trunked via 802.1Q.

## 5.2 Key Configuration Decisions

- RFC1918 and bogon blocking disabled on WAN — required because WAN operates on a private VMware NAT subnet, not a public IP.

- All VLAN interfaces use static IPv4 addressing (.1 in each subnet) as the default gateway.

- DHCP scopes use .100–.199 ranges, reserving .2–.99 for static device assignments.

- DNS resolver set to 8.8.8.8 / 8.8.4.4 for reliable name resolution across all VLANs.

- Firewall rules use subnet objects (e.g. TRUSTED subnets) rather than hardcoded IPs for maintainability.

# 6. Skills Demonstrated

| Skill Area | Evidence |
| --- | --- |
| Network segmentation | Designed and implemented 4-VLAN architecture with distinct security zones |
| Firewall rule design | Built stateful firewall rules with default-deny posture and documented justifications |
| DHCP administration | Configured independent DHCP scopes per VLAN with appropriate ranges |
| pfSense / FreeBSD | Installed, configured, and administered pfSense CE 2.8.1 from scratch |
| VMware networking | Configured virtual network adapters, trunk ports, and Host-only networking |
| Security reasoning | Applied principles of least privilege, blast radius containment, and zero trust |
| Technical documentation | Produced structured network documentation suitable for professional portfolio |

End of Document